### Topic 1: Microarchitecture - Introduction

To understand how a computer truly works, we have to bridge the gap between the programmer's view (the architecture or instruction set) and the underlying digital logic (the gates and circuits). This connection is called microarchitecture 1, 2.

While an architecture (like RISC-V, ARM, or Intel x86) defines the instructions and registers available to a programmer, the microarchitecture is the specific, physical arrangement of hardware building blocks—such as registers, ALUs (Arithmetic Logic Units), multiplexers, and memories—needed to actually execute those instructions 1-3.

Because there are many ways to build a processor, a single architecture can be implemented using many different microarchitectures, each offering different trade-offs in terms of performance, cost, complexity, and power consumption 2, 3. For example, a designer might choose a simple single-cycle design, a cost-effective multicycle design, or a high-performance pipelined design 4.

No matter the approach, the design of a microarchitecture is typically divided into two interacting parts:

- The Datapath: This is the hardware that actually operates on words of data. It contains the state elements (like the program counter and register files) as well as the combinational logic (like the ALU and multiplexers) used to perform calculations 5.

- The Control Unit: This acts as the brain of the processor. It receives the current instruction from the datapath, decodes it, and generates the specific control signals (such as read/write enables and multiplexer selects) that tell the datapath exactly how to execute that instruction 5.

At the heart of all this is the architectural state, which is the information necessary to completely define what the computer is doing at any given moment. This includes the Program Counter (PC), the 32 registers, and memory 6, 7. On each clock cycle, the processor reads the current architectural state, executes an instruction, and updates the hardware to produce a new architectural state 7.

Take a moment to review this introduction to microarchitecture. Whenever you are ready, just say "yes" or "next" and I will explain Topic 2: Performance analysis.

### Topic 2: Performance Analysis

While marketing departments often highlight clock frequency or the number of processor cores, the only gimmick-free way to measure a computer's performance is by measuring the actual execution time of a program 1, 2. Because measuring every possible program you might run is impossible, computer architects rely on benchmarks, which are standardized collections of representative programs (such as Dhrystone, CoreMark, or SPEC) used to evaluate and compare processor performance 2-4.

At the microarchitecture level, performance analysis relies on a fundamental equation to calculate a program's execution time:

Execution Time = (Number of Instructions) × (Cycles per Instruction) × (Seconds per Cycle) 5.

Here is a breakdown of how a processor's design impacts each component of this equation:

- Number of Instructions: This is determined by the program, the compiler's cleverness, and the instruction set architecture itself 5. If we are running a known program on a specific architecture (like RISC-V), the number of instructions remains constant, completely independent of the underlying microarchitecture 5.

- Cycles per Instruction (CPI): This represents the average number of clock cycles required to execute a single instruction, and it is the reciprocal of the processor's throughput (Instructions Per Cycle, or IPC) 5. Different microarchitectures—such as single-cycle, multicycle, or pipelined designs—will yield radically different CPIs 5.

- Seconds per Cycle (Clock Period, $T_c$): This is the duration of a single clock cycle 6. The clock period is strictly determined by the processor's critical path—the longest, slowest delay a signal faces traveling through the physical logic gates 6.

The ultimate challenge for a microarchitect is to choose a design that minimizes the overall execution time by finding the perfect balance between CPI and the clock period, while simultaneously satisfying tight constraints on hardware cost and power consumption 7.

Take a moment to review this performance equation. Whenever you are ready, just say "yes" or "next" and I will explain Topic 3: Single-Cycle Processor - Single Cycle Datapath.

### Topic 3: Single-Cycle Processor - Single Cycle Datapath

A single-cycle processor is a microarchitecture designed to execute an entire instruction from start to finish in just one clock cycle 1. To build it, designers construct a datapath by connecting architectural state elements (like the program counter, register file, and memory) with combinational logic (like ALUs, adders, and multiplexers) 2.

Because the processor must accomplish everything in one cycle, the datapath is built to compute multiple possibilities in parallel and then use multiplexers to pick the correct result 3. Here is how the datapath is structured to handle different types of instructions:

- Instruction Fetch: Every instruction begins with the Program Counter (PC), which holds the address of the current instruction 4. The PC points to the instruction memory, which fetches the 32-bit instruction 4. Simultaneously, an adder computes PC + 4 (since instructions are 4 bytes long) to prepare the address of the next sequential instruction 5.

- Memory Instructions (e.g., Load/Store): For a load word (lw) instruction, the datapath reads a base address from the register file 6. It then uses an Extend unit to sign-extend the instruction's 12-bit immediate offset to 32 bits 6. The Arithmetic Logic Unit (ALU) adds the base address and the sign-extended offset to find the exact memory address 6. The memory reads the data, and it is written back into the destination register 7. A store word (sw) behaves similarly but reads a second register to get the data that will be written into the data memory 8.

- R-Type Instructions (e.g., add, sub, and, or): These instructions operate purely on registers. The datapath reads two source registers from the register file, uses the ALU to perform the specified operation, and routes the ALU's output directly back to the destination register 9. A multiplexer in front of the ALU determines whether the ALU's second operand should be a register value (for R-type instructions) or a sign-extended immediate (for load/store instructions) 10.

- Branch Instructions (e.g., branch if equal - beq): To handle a branch, the ALU performs a subtraction on the two source registers; if the ALU's "Zero" flag triggers, the registers are equal 11, 12. Meanwhile, a separate dedicated adder computes the branch target address by adding the current PC to the sign-extended branch offset 12. A multiplexer then selects whether the next PC should be PC + 4 or the calculated branch target address, depending on the ALU's Zero flag and the control unit 12.

Take a moment to review the single-cycle datapath. Whenever you are ready, just say "yes" or "next" and I will explain Topic 4: Single Cycle Control!

### Topic 4: Single-Cycle Processor - Single Cycle Control

Now that the datapath is physically wired to handle data, the processor needs a "brain" to tell those components exactly what to do for each specific instruction. This is the job of the control unit, which computes the control signals based on the instruction's fields—specifically the opcode (op), funct3, and funct7 fields 1.

To simplify the design, the control unit is typically decomposed hierarchically into two main blocks: the Main Decoder and the ALU Decoder 2.

- Main Decoder: This block looks at the instruction's 7-bit opcode to determine the general type of instruction being executed (e.g., load, store, R-type, branch) 3. It generates most of the control signals that route data through the datapath, such as multiplexer select signals, memory write enables, and register write enables 2, 3. It also produces a 2-bit internal signal called ALUOp (ALU Operation) to guide the ALU Decoder 3.

- ALU Decoder: This block is responsible for determining the specific arithmetic or logical operation the ALU needs to perform and outputs a multi-bit ALUControl signal to the ALU 3. It figures this out by combining the ALUOp signal from the Main Decoder with the instruction's funct3 field (and sometimes specific bits from the funct7 and op fields) 3.

Here is how the ALU Decoder interprets the ALUOp signal:

- An ALUOp of 00 indicates that the ALU should perform an addition. This is used by load and store instructions to calculate memory addresses 4.

- An ALUOp of 01 indicates that the ALU should perform a subtraction. This is used by branch instructions (like beq) to compare two registers 4.

- An ALUOp of 10 indicates an R-type instruction. Because R-type instructions can perform many different operations, the ALU Decoder must look at the funct3 field to decide whether it needs to command the ALU to add, sub, and, or, slt (set less than), etc. 4.

Take a moment to review single-cycle control. Whenever you are ready, just say "yes" or "next" and I will move on to the next major section: Topic 5: Pipelined Processor - Pipelined Data Path.

### Topic 5: Pipelined Processor - Pipelined Data Path

To significantly improve the throughput of a digital system, designers use instruction pipelining, a concept very similar to a manufacturing assembly line 1, 2. The pipelined datapath is created by subdividing the single-cycle datapath into multiple stages separated by pipeline registers 3. These registers are critical because they prevent data from one pipeline stage from catching up with and corrupting the data in the next stage 4.

In a classic five-stage RISC pipeline, up to five instructions can execute simultaneously, with one instruction residing in each stage 2. The five stages are:

- Fetch: The processor reads the instruction from the instruction memory 5.

- Decode: The processor reads the source operands from the register file and decodes the instruction to produce the necessary control signals 5.

- Execute: The processor performs the required computation using the Arithmetic Logic Unit (ALU) 5.

- Memory: The processor reads from or writes to the data memory, if applicable 5.

- Writeback: The processor writes the final result back to the register file, if the instruction requires it 5.

Because each stage contains only a fraction of the entire processor's logic, the clock frequency can be driven much faster than in a single-cycle design 2. One of the most important rules of this design is that all signals associated with a particular instruction—including data and the control signals generated during the Decode stage—must advance through the pipeline registers in unison so that they remain perfectly synchronized with the instruction as it moves down the line 6, 7.

Take a moment to review the Pipelined Data Path. Whenever you are ready, just say "yes" or "next" and I will explain Topic 6: Pipelined Control: Hazards.

Here is a detailed breakdown of Topic 6: Pipelined Control: Hazards.

In a pipelined system, multiple instructions are handled concurrently to improve throughput 1. However, a pipeline hazard occurs when a pipeline, or a portion of it, must stall because conditions do not permit continued execution 2. This typically happens when one instruction is dependent on the results of another instruction that has not yet completed 1. When a pipeline is stalled, it introduces an unused stage that propagates through the system, which behaves like a no-operation and is referred to as a pipeline bubble 2, 3.

There are three main categories of pipeline hazards: resource, data, and control 2.

### 1. Resource Hazards (Structural Hazards)

A resource hazard occurs when two or more instructions that are already in the pipeline compete for the exact same hardware resource at the same time 2, 4. For example, if the main memory only has a single port, the processor cannot perform an instruction fetch and a data read/write in parallel 5. The primary solution to a resource hazard is to increase the available hardware, such as adding multiple memory ports or multiple Arithmetic Logic Units (ALUs) 6.

### 2. Data Hazards

A data hazard happens when there is a conflict in accessing an operand location 7. This generally occurs when an instruction attempts to read from a register that has not yet been written back to by a previous instruction 8. Without special treatment, the pipeline will compute the wrong result 1. There are three specific types of data hazards 9:

- Read after write (RAW) or true dependency: A succeeding instruction attempts to read a location before a prior instruction has finished writing to it 9.

- Write after read (WAR) or antidependency: A succeeding instruction writes to a location before a prior instruction has finished reading from it 9.

- Write after write (WAW) or output dependency: Two instructions both attempt to write to the same location, but the writes occur in the reverse order of the intended sequence 9.

To resolve data hazards, computer architects use several techniques:

- Forwarding (or Bypassing): This technique resolves hazards without slowing down the pipeline by adding multiplexers in front of the ALU. It forwards a result directly from the Memory or Writeback stage to a dependent instruction waiting in the Execute stage 10.

- Stalling: Sometimes forwarding is not enough. For example, a load word instruction (lw) does not finish reading data until the end of the Memory stage, creating a two-cycle latency 11. The dependent instruction must be stalled, holding up the pipeline until the data is available 12.

- Software Interlocks: A programmer or compiler can manually insert nop (no-operation) instructions between dependent instructions so that the result has time to reach the register file before the next instruction reads it 13, 14. However, this method complicates programming and degrades performance 14.

### 3. Control Hazards (Branch Hazards)

A control hazard occurs when the pipeline needs to fetch the next instruction, but the decision of which instruction to fetch has not yet been made (such as when waiting for a conditional branch to resolve) 8, 15. If the pipeline fetches the wrong instructions, they must subsequently be discarded 16.

To handle control hazards, processors use strategies like:

- Stalling: The processor can simply stall the pipeline until the branch decision is definitively calculated, though this severely degrades system performance if branches occur often 17.

- Branch Prediction: The processor guesses whether the branch will be taken and begins executing instructions based on that prediction 18. If the prediction is later determined to be wrong, the incorrect instructions are flushed (cleared) from the pipeline registers 18, 19. These wasted instruction cycles are known as a branch misprediction penalty 18.

### Topic 7: Solving Data/Control Hazards

To ensure a pipelined processor computes the correct results without severe performance degradation, hardware and software techniques are used to resolve pipeline hazards.

Solving Data HazardsA data hazard occurs when an instruction attempts to read a register that has not yet been written back by a previous instruction 1. Computer architects solve these using several techniques:

- Forwarding (or Bypassing): This solves many data hazards without slowing down the pipeline 2. It involves adding multiplexers in front of the ALU to select operands directly from the results currently in the Memory or Writeback stages, rather than waiting for them to be written to the register file 2, 3.

- Stalling (Pipeline Bubbles): Sometimes forwarding is insufficient. For example, a load word (lw) instruction does not finish reading data until the end of the Memory stage, creating a two-cycle latency 4. If the immediate next instruction needs this data, it cannot be forwarded in time. The processor must stall the dependent instruction, effectively inserting an unused stage called a bubble (which acts like a nop instruction) that propagates through the pipeline until the data is available 5, 6.

- Software Interlocks: Alternatively, a compiler can resolve data hazards by manually inserting nop instructions between dependent instructions so the result has time to reach the register file, though this degrades overall performance 1, 7.

Solving Control (Branch) HazardsA control hazard occurs because the processor does not know which instruction to fetch next until a branch decision is fully evaluated 8. This is resolved using:

- Stalling: The simplest hardware mechanism is to stall the pipeline until the branch decision is calculated 9. However, because branches occur frequently, this can severely degrade system performance 9.

- Branch Prediction: Instead of stalling, the processor predicts whether the branch will be taken and speculatively begins fetching and executing instructions based on that guess 10. If the prediction is later determined to be wrong, the hardware must flush (discard) the incorrect instructions from the pipeline registers by clearing their contents 10, 11. The wasted cycles resulting from a wrong guess are referred to as the branch misprediction penalty 10.

Take a moment to review these hazard solutions. Whenever you are ready, just say "yes" or "next" and I will explain the final topic on your list: Performance Analysis for pipelined processors.

### Topic 8: Performance Analysis (Pipelined Processors)

To evaluate the performance of a pipelined processor, computer architects analyze both the Cycles Per Instruction (CPI) and the overall cycle time.

Ideal vs. Actual CPIIdeally, a pipelined processor issues a new instruction every single clock cycle, which would result in a perfect CPI of 1 1. In reality, however, the average CPI is always slightly higher than 1 1. This occurs because pipeline hazards require the processor to insert stalls or flush instructions from the pipeline 1. For example, a load instruction might take an extra cycle if the processor has to stall for a data dependency, and a mispredicted branch might waste several cycles because incorrect instructions must be flushed 2. Because of these penalties, the actual average CPI depends heavily on the specific mix of instructions in the program being executed 1, 2.

Cycle Time and LimitationsWhile pipelining vastly improves instruction throughput, the cycle time is limited by physical hardware constraints:

- The Critical Path: The overall clock cycle time of a pipelined processor is dictated by the critical path, which is the single slowest stage in the pipeline 3. If the pipeline stages are not perfectly balanced with equal amounts of logic, the clock must still be slowed down to accommodate the longest stage. This imbalanced delay inherently decreases the theoretical benefits of pipelining 4.

- Sequencing Overhead: Another major factor that limits pipelining performance is sequencing overhead 4. Every time you divide the processor into separate stages using pipeline registers, you introduce hardware delays (specifically, the register setup times and clock-to-Q delays). In a pipelined processor, this timing overhead applies to every single pipeline stage, rather than just being paid once for the overall datapath 4.

This completes your list of topics on Microarchitecture and Pipelined Processors! You have now successfully covered single-cycle datapaths and control, pipelined datapaths, hazards, and performance analysis.

What would you like to study next? Your notes cover a vast amount of material, including Memory Systems (Caches and Virtual Memory), I/O Systems, Superscalar Processors, Multicore Computers, and the IA-64 Architecture.